- FREEDOM OF SPEECH -

or lets finish some of the multi- sega kit.......................

Fred Konopaska

aka juice28

I don't know about most of the individuals that installed Clay's Multi-Sega kit, but I like it for the most part. After playing for about 10 minutes though I realized something had to be done about the speech board at the least. Since I had a Star Trek speech board to begin with the other games (Zector and Space Fury) would ramble Star Trek garbage that made no sense. Although my son and I got a kick out if it for about 2 minutes........

I wanted to try to finish this kit a little more then it was and not have to swap roms to hear the right speech on different games.

So begins the multi-sega kit speech board hack ;)

What this hack will do-

This hack will make your speech board automatically load the correct code for the games that use the speech PCB. These include Star Trek, Space  Fury and Zector. NOTE because the sound board is not reset between games it may occasionally hang when switching games but reseting to the menu and trying again clears it up. I am working on a simple fix to reset the sound board indivual of the CPU board between games to clear this up.

What this hack will NOT do-

This hack does not address the issue of replacing or adding the different sound PCB's such as the Universal Sound board, Elim/Zec sound board and so on. I'll leave this for another hack/upgrade doc.

NOTE- If you do this hack and mess up your speech pcb, game or yourself don't blame me. If you can't handle this hack then please don't do it.

Parts needed for this hack -

27C010 Eprom (or equiv 128k*8 eprom)

27C32  Eprom (or equiv 4k*8 eprom)

Combined rom images (or the ability to combine them)

Wire, ability to soldier and read a schematic.

Remove u7,u6,u5,u4 and u3 from you sega speech pcb.

MAKE SURE YOUR SEGA SPEECH BOARD IS JUMPERED FOR 2732 EPROMS IN THE U3, U4, U5 AND U6 POSITIONS.

U7 MUST BE JUMPERED FOR 2716.

Refer to the schematics to be sure. If your pcb has a 2716 in U7 and the rest are 2732's then your okay.

Lets dig into it.

Looking at the doc's that came with the Multi Sega, Clay has added the ability to bank switch sound boards and speech boards. This is a great plus as without this latch byte this hack would be impossible without redesign of the whole kit and code.....He actually has two 1 byte latches but we are only concerned with latch 1.

Info from Multi-Sega installation manual-

LATCH1-

D7654 3210			SOUND BOARD

STAR TREK		   0001 0000                    (USB)

ELIMINATOR	             0010 0001			(ELIM/ZEK)

SPACE FURY		   0100 0010			(SF)

TAK/SCAN			   0001 0011			(USB)

ZECTOR		             0010 0100			(ELIM/ZEK)

ELIM 2P			   0010 0101			(ELIM/ZEK)

ELIM 4P			   0010 0110			(ELIM/ZEK)

SF (ORIGINAL)		   0100 0111			(SF)

If we disregard the top bits D5-D7 and just focus on the 3 lower bits we can derive this.

LATCH1			D210

STAR TREK		  000

SPACE FURY		  010

ZECTOR			  100

SF (ORIGINAL)		  111

These are the bit patterns we need to enable the different roms for each of the games that use speech. At first glance it looks unfriendly but if you think about it there is a way around it. You could either use some crazy logic to decode these four different 3 bit patterns then bank switch the eprom with that or you can do it the easy way (ilbiet wasteful). Since eproms are cheap I am taking the easy way.

Star Trek and Zector use the same code in the U7 eprom. Space Fury uses different code in U7. U7 is a 27C16 in both games so if we burn a 27C32 with Star Trek in the bottom half and Space Fury in the top half and use the A11 pin we can bank switch with D1 of latch1.

So go ahead and burn a 27C32 with the combined Star Trek and Space Fury U7 Roms or my file.

Bend pin 21 (A11) up on the 27C32 and plug it into the U7 socket on your speech board. Run pin 21 (A11 of 27C32)  to D1 of latch1 on the multi sega.

Now combine and burn a 27C010 eprom according to the table below or use the code I combined ;)

Bend up pins 1,2,3,4,22,24,28,29,30,31 and 32 on the 27C010.

Place the 27c1001 into the U5 socket. Line the bottom of the 27C010 up with the bottom of the socket so the top stick over the socket.

1,32,31 goto +5v someplace on the speech board

22,24 together to pin 8 of u14 on the speech board

4 to pin 35 of u14

28 to pin 36 of u14

29 to D0 latch1 on the multi-kit

3 to D1 latch1 on the multi-kit

2 to D2 latch1 on the multi-kit

Put you pcb back in and give it a try.

Thats if for now. I'll compile the roms and go through these while installing this hack to be sure all is correct. If you find errors please let me know. If you damage your pcbs using this hack your on your own. If you think of an easy way to reset just the sound board between games give me an email.

Fred

AKA JUICE28

Arcadexpo@mchsi.com

--------------------------------------------------------------------

SPACE FURY SPEECH ROM NUMBERING

U3	N/A

U4	972	2732

U5	971	2732

U6	970	2732

U7	808	2716

STAR TREK SPEECH ROM NUMBERING

U3	N/A

U4	N/A

U5	1872	2732

U6	1871	2732

U7	1607	2716

ZECTOR SPEECH ROM NUMBERING

U3	N/A

U4	1610	2732

U5	1609	2732

U6	1608	2732

U7	1607	2716

------------------------------------------------------------------

37C32 MEMORY LAYOUT 4KX8 EPROM

000	U7 STAR TREK/ZECTOR SPEECH ROM

7FF

800	U7 SPACE FURY SPEECH ROM

FFF

------------------------------------------------------------------

27C1001 MEMORY LAYOUT 128KX8 EPROM

STAR TREK SPEECH ROMS

00000	U6

00FFF

01000	U5

01FFF

02000	U4 OR FILL

02FFF

03000	U3 OR FILL

03FFF

----------

STAR TREK FILLER

04000	U6

04FFF

05000	U5

05FFF

06000	U4 OR FILL

06FFF

07000	U3 OR FILL

07FFF

---------

SPACE FURY SPEECH ROMS

08000	U6

08FFF

09000	U5

09FFF

0A000	U4

0AFFF

0B000	U3 OR FILLER

0BFFF

---------

SPACE FURY FILLER

0C000	U6

0CFFF

0D000	U5

0DFFF

0E000	U4

0EFFF

0F000	U3 OR FILLER

0FFFF

----------

ZECTOR SPEECH ROMS

10000	U6

10FFF

11000	U5

11FFF

12000	U4

12FFF

13000	U3 OR FILLER

13FFF

---------

STAR TREK FILLER

14000	U6

14FFF

15000	U5

15FFF

16000	U4 OR FILL

16FFF

17000	U3 OR FILL

17FFF

---------

SPACE FURY FILLER

18000	U6

18FFF

19000	U5

19FFF

1A000	U4

1AFFF

1B000	U3 OR FILLER

1BFFF

----------

SPACE FURY SPEECH ROMS

1C000	U6

1CFFF

1D000	U5

1DFFF

1E000	U4

1EFFF

1F000	U3 OR FILLER

1FFFF